Question
Write an Embedded C application integrating ADC, UART, and GPIO using structured programming

Aim

To develop an Embedded C application using structured programming that integrates ADC (Analog-to-Digital Converter) for analog signal acquisition, UART (Universal Asynchronous Receiver/Transmitter) for serial communication, and GPIO (General Purpose Input/Output) for controlling input/output devices, enabling efficient data acquisition, processing, and communication in an embedded system.

Requirements Analysis

Functional Requirements

- Initialize the ADC, UART, and GPIO peripherals.

- Read analog input values using the ADC.

- Convert the analog signal into a digital value.

- Transmit ADC data through UART.

- Receive commands or data through UART.

- Control GPIO devices (LED/Switch) based on received data or ADC values.

- Display ADC readings and communication status.

- Detect communication errors and handle invalid data gracefully.

Non-Functional Requirements

- Reliability: Ensure accurate ADC conversion and dependable UART communication.

- Real-Time Performance: Read and transmit data with minimal delay.

- Efficiency: Minimize CPU and memory usage during execution.

- Maintainability: Use structured and modular programming for easy modification.

- Scalability: Support the addition of more ADC channels and GPIO devices.

- Portability: The Embedded C program should be adaptable to different microcontrollers.

- Robustness: Handle communication errors and invalid inputs without system failure.

- Low Power Consumption: Optimize peripheral usage for energy-efficient operation.

Block Diagram

Algorithm

- Initialize the ADC, UART, and GPIO peripherals.

- Read the analog input using the ADC.

- Convert the analog signal into a digital value.

- Display the ADC value.

- Transmit the ADC value through UART.

- Receive data or commands through UART.

- Control the GPIO (LED) based on the received command or ADC value.

- Repeat the process continuously.

Embedded C Program

#include <stdio.h>

int adcValue = 512;

char uartData = '1';

void ADC_Init()

{

printf("ADC Initialized\n");

}

int ADC_Read()

{

return adcValue;

}

void UART_Init()

{

printf("UART Initialized\n");

}

void UART_Send(int data)

{

printf("UART TX : ADC Value = %d\n", data);

}

char UART_Receive()

{

return uartData;

}

void GPIO_Control(char data)

{

if(data == '1')

printf("LED ON\n");

else

printf("LED OFF\n");

}

int main()

{

int value;

ADC_Init();

UART_Init();

value = ADC_Read();

printf("ADC Value : %d\n", value);

UART_Send(value);

uartData = UART_Receive();

GPIO_Control(uartData);

return 0;

}

Sample Output

ADC Initialized

UART Initialized

ADC Value : 512

UART TX : ADC Value = 512

LED ON

TEST CASE

| Test Case | Expected Result | Status |
| --- | --- | --- |
| Valid ADC Input | ADC value displayed | Pass |
| UART Transmission | ADC value transmitted | Pass |
| UART Receive = '1' | LED ON | Pass |
| UART Receive = '0' | LED OFF | Pass |

Advantages

- Simple and modular program structure.

- Efficient integration of ADC, UART, and GPIO.

- Easy to understand and maintain.

- Suitable for real-time embedded applications.

- Easily expandable for additional peripherals.

Conclusion

The Embedded C application successfully integrates ADC, UART, and GPIO using a structured programming approach. The system acquires analog data, communicates it through UART, and controls GPIO devices based on received commands, making it suitable for embedded monitoring and control applications.

Question

- Design an Embedded C firmware for an industrial HVAC controller integrating sensors and actuators with modular code organization.

Aim

To design and develop an Embedded C firmware for an Industrial HVAC (Heating, Ventilation, and Air Conditioning) Controller that integrates temperature and humidity sensors with actuators such as fans and cooling/heating units using a modular code organization, enabling efficient environmental monitoring and automatic climate control.

Requirements Analysis

Functional Requirements

- Read temperature and humidity sensor data.

- Monitor environmental conditions continuously.

- Control cooling, heating, and ventilation actuators.

- Turn actuators ON/OFF based on preset threshold values.

- Display the current sensor readings and actuator status.

- Ensure safe operation by preventing abnormal temperature conditions.

Non-Functional Requirements

- Reliable environmental monitoring.

- Fast response to changing temperature and humidity.

- Modular and maintainable Embedded C code.

- Low power consumption.

- Scalable for additional sensors and actuators.

Block Diagram

Algorithm

- Initialize the temperature and humidity sensors.

- Read the sensor values.

- Compare the values with predefined threshold limits.

- If the temperature is high, turn ON the cooling fan.

- If the temperature is low, turn ON the heater.

- Control the ventilation motor based on humidity level.

- Display the sensor readings and actuator status.

- Repeat the process continuously.

Embedded C Program

#include <stdio.h>

int temperature = 30;

int humidity = 65;

void ReadSensors()

{

printf("Temperature = %d C\n", temperature);

printf("Humidity = %d %%\n", humidity);

}

void ControlHVAC()

{

if(temperature > 28)

printf("Cooling Fan : ON\n");

else

printf("Cooling Fan : OFF\n");

if(temperature < 20)

printf("Heater : ON\n");

else

printf("Heater : OFF\n");

if(humidity > 60)

printf("Ventilation : ON\n");

else

printf("Ventilation : OFF\n");

}

int main()

{

printf("Industrial HVAC Controller\n\n");

ReadSensors();

ControlHVAC();

return 0;

}

Sample Output

Industrial HVAC Controller

Temperature = 30 C

Humidity = 65 %

Cooling Fan : ON

Heater : OFF

Ventilation : ON

Test Cases

| Test Case | Expected Result | Status |
| --- | --- | --- |
| Temperature > 28°C | Cooling Fan ON | Pass |
| Temperature < 20°C | Heater ON | Pass |
| Humidity > 60% | Ventilation ON | Pass |
| Normal Conditions | Appropriate actuator operation | Pass |

Advantages

- Automatic climate control.

- Improved energy efficiency.

- Modular and reusable code.

- Reliable industrial operation.

- Easy integration with additional sensors.

Conclusion

The Embedded C firmware successfully implements an Industrial HVAC Controller using a modular code organization. The system continuously monitors temperature and humidity, controls the cooling fan, heater, and ventilation unit automatically, and provides an efficient and reliable solution for industrial environmental control.